The University of Azad Jammu and Kashmir

The University of Azad Jammu and Kashmir

Name: Sakeena bano

Roll NO: 2024-SE-29

Course Title: Computer network

Submitted To: Mam Asma

Department of Software Engineering

Lab 3:

Tester – Detailed Explanation

This device is a multi-purpose cable tester used to check the condition and wiring of different types of cables, including RJ-45 (Ethernet cables), RJ-11 (telephone cables), USB cables, and BNC coaxial cables. It is an essential tool for network technicians, electricians, and IT professionals to diagnose cable problems quickly and accurately.

Purpose of the Cable Tester

The main purpose of this tester is to identify whether a cable is:

Properly connected

Short-circuited

Incorrectly wired

Non-parallel

Disconnected

Faulty or damaged

It helps reduce troubleshooting time in networking and communication systems.

Main Features and Functions

- Supported Cable Types

This tester can analyze:

RJ-45 – Used for Ethernet and networking cables

RJ-11 – Used for telephone lines

USB – Used for computer peripherals and charging

BNC – Used for coaxial cables in CCTV, broadcasting, and radio systems

Indicator Lights Explanation

Top LED Sequence (Pin/Wire Condition)

The numbered LEDs at the top represent cable pin connections.

They check whether each wire inside the cable is correctly aligned.

Left Side Indicators for RJ-45 / RJ-11 / USB

SHORT

Indicates that two or more wires are touching incorrectly

Means internal wiring fault

Cable may not function properly

CONNECTED

Indicates proper wiring

All wires are correctly connected

Cable is working normally

NON-PARALLEL

Indicates wires are crossed or arranged incorrectly

Cable may have incorrect pinout

NO CONNECTION

Indicates broken or disconnected wires

Cable is damaged or incomplete

Right Side Indicators for BNC Cables

SHORT

Internal short circuit detected

25 OHM

Cable termination resistance is 25 ohms

50 OHM

Proper 50-ohm termination detected

Common for many coaxial systems

NO TERMINATOR

No termination resistance found

Cable may be open-ended or improperly installed

Battery Indicators

BATTERY GOOD

Green light indicates sufficient battery power

BATTERY LOW

Yellow light indicates battery replacement is needed soon

Working Principle

Connect one end of the cable to the tester.

Attach the remote end if required.

Press the test button.

Observe LED indicators.

Determine cable status based on lights.

Advantages

Easy to use

Fast fault detection

Supports multiple cable types

Portable design

Useful for field technicians

Saves maintenance time

Applications

Computer networking

Telephone installations

CCTV systems

USB troubleshooting

Cable manufacturing quality checks

Home and office wiring inspections

Conclusion

The RJ-45/RJ-11/USB/BNC cable tester is a versatile diagnostic tool designed to ensure proper cable functionality and detect faults such as shorts, miswiring, or disconnections. Its multiple indicators and broad cable compatibility make it highly valuable for professional and personal use in communication and networking systems.

Diagram